SMART DEVICE

HTTP API Reference

ESP32 Air Quality Monitor

Version 1.0

# 1. Overview

The Smart Device exposes a small HTTP API hosted directly on the ESP32 microcontroller. The API is used internally by the built-in web dashboard, but it is fully open and can be consumed by any HTTP client — Python, JavaScript, curl, Postman, mobile apps, or other embedded devices.

There is no authentication, no API key, and no rate limiting. Any client connected to the ESP32 access point can call the endpoints.

## 1.1 Base URL

| Access Method | URL |
| --- | --- |
| mDNS (recommended) | http://smartdevice.local |
| Direct IP (fallback) | http://192.168.4.1 |

Both URLs resolve to the same ESP32. Use the mDNS name on macOS, iOS, and most desktop Linux distributions. Use the IP fallback on Windows without Bonjour or on Android devices where mDNS does not resolve.

## 1.2 Connection Requirements

- Client must be connected to the smartdevice Wi-Fi network (password: 12345678).

- ESP32 supports a maximum of 4 simultaneous clients.

- No internet connection is required — the API is local-only.

## 1.3 Protocol Summary

| Property | Value |
| --- | --- |
| Protocol | HTTP/1.1 |
| Port | 80 |
| Encryption | None (plain HTTP, local network only) |
| Authentication | None |
| Rate limiting | None |
| CORS | Open — no restrictions |
| Response format | JSON (application/json) and HTML (text/html) |

# 2. Endpoints

The API exposes two endpoints. Both accept the HTTP GET method only. Any other method returns 404 Not Found.

## 2.1 Endpoint Summary

| Method | Path | Description |
| --- | --- | --- |
| GET | / | Returns the dashboard HTML page |
| GET | /data | Returns the latest sensor readings as JSON |

## 2.2 GET /

Returns the complete dashboard web page (HTML + embedded CSS + JavaScript). This is the URL a user opens in a browser to see the live cards and CSV table.

### Request

GET / HTTP/1.1

Host: smartdevice.local

### Response

| Property | Value |
| --- | --- |
| Status | 200 OK |
| Content-Type | text/html |
| Body | Full HTML document for the dashboard UI |

## 2.3 GET /data

Returns the most recent sensor readings as a JSON object. This is the endpoint used by the dashboard's JavaScript to refresh the live cards every 2 seconds, and by any external client (Python, mobile app, etc.) that wants real-time data.

### Request

GET /data HTTP/1.1

Host: smartdevice.local

Accept: application/json

No query parameters, no request body, no headers required.

### Response

| Property | Value |
| --- | --- |
| Status | 200 OK |
| Content-Type | application/json |
| Cache-Control | no-store |

### Response Body — Example

{

"temp": 27.40,

"hum": 64.00,

"mq7": 100.62,

"mq135": 47.51,

"mq7_raw": 412,

"mq135_raw": 389

}

### Response Body — Field Reference

| Field | Type | Unit | Range | Description |
| --- | --- | --- | --- | --- |
| temp | float | °C | 0 – 50 | Ambient temperature (DHT11) |
| hum | float | % RH | 20 – 90 | Relative humidity (DHT11) |
| mq7 | float | ppm | 0 – 1000 | Estimated CO concentration (MQ-7) |
| mq135 | float | ppm | 0 – 500 | Estimated air quality index (MQ-135) |
| mq7_raw | int | — | 0 – 4095 | Raw 12-bit ADC value (MQ-7) |
| mq135_raw | int | — | 0 – 4095 | Raw 12-bit ADC value (MQ-135) |

### Behaviour Notes

- temp and hum return 0 (never null or NaN) if the DHT11 fails to respond.

- The ESP32 refreshes its internal sensor cache once per second. Polling faster than that simply returns the previously cached values.

- Each call returns a fresh JSON object — no streaming, no WebSocket, no Server-Sent Events. Use polling.

- Conversion formulas applied to the raw ADC values: mq7 = (mq7_raw / 4095) × 1000 and mq135 = (mq135_raw / 4095) × 500.

# 3. Status Codes & Errors

| Code | Meaning | When it occurs |
| --- | --- | --- |
| 200 OK | Success | Normal response from both endpoints |
| 404 Not Found | Not Found | Any path other than / or /data, or any method other than GET |

Network-level failures (connection refused, timeout, no route to host) are not produced by the ESP32 itself — they indicate that the client is not connected to the smartdevice Wi-Fi network or the ESP32 is powered off.

# 4. Client Examples

Minimal, copy-pasteable examples for the most common clients. All examples assume the client is already connected to the smartdevice Wi-Fi network.

## 4.1 curl

# One-shot reading

curl http://smartdevice.local/data

# Pretty-printed

curl -s http://smartdevice.local/data | jq

# Poll every 2 seconds

while true; do

curl -s http://smartdevice.local/data

echo

sleep 2

done

## 4.2 React (Hooks)

import { useEffect, useState } from "react";

const URL = "http://smartdevice.local/data";

export default function SmartDeviceMonitor() {

const [data, setData]   = useState(null);

const [error, setError] = useState(null);

useEffect(() => {

let cancelled = false;

async function fetchData() {

try {

const res = await fetch(URL);

if (!res.ok) throw new Error("HTTP " + res.status);

const json = await res.json();

if (!cancelled) { setData(json); setError(null); }

} catch (e) {

if (!cancelled) setError(e.message);

}

}

fetchData();                                 // initial call

const id = setInterval(fetchData, 2000);     // poll every 2s

return () => { cancelled = true; clearInterval(id); };

}, []);

if (error) return <div>Error: {error}</div>;

if (!data) return <div>Loading...</div>;

return (

<div>

<h2>Smart Device — Live Readings</h2>

<p>Temperature : {data.temp.toFixed(1)} °C</p>

<p>Humidity    : {data.hum.toFixed(1)} %</p>

<p>MQ-7  (CO)  : {data.mq7.toFixed(2)} ppm</p>

<p>MQ-135 (AQ) : {data.mq135.toFixed(2)} ppm</p>

</div>

);

}

## 4.3 React — Custom Hook (reusable)

import { useEffect, useState } from "react";

export function useSmartDevice(intervalMs = 2000) {

const [data, setData]   = useState(null);

const [error, setError] = useState(null);

useEffect(() => {

let cancelled = false;

const tick = async () => {

try {

const r = await fetch("http://smartdevice.local/data");

const j = await r.json();

if (!cancelled) { setData(j); setError(null); }

} catch (e) {

if (!cancelled) setError(e);

}

};

tick();

const id = setInterval(tick, intervalMs);

return () => { cancelled = true; clearInterval(id); };

}, [intervalMs]);

return { data, error };

}

// Usage in any component:

//   const { data, error } = useSmartDevice(2000);

## 4.4 JavaScript (vanilla, browser or Node 18+)

// Single reading (modern fetch API)

const data = await fetch("http://smartdevice.local/data").then(r => r.json());

console.log(data);

// Polling every 2 seconds

setInterval(async () => {

try {

const r = await fetch("http://smartdevice.local/data");

const d = await r.json();

console.log(`T=${d.temp} H=${d.hum} CO=${d.mq7} AQ=${d.mq135}`);

} catch (e) {

console.error("Read failed:", e);

}

}, 2000);

## 4.5 PowerShell

# Single reading

Invoke-RestMethod -Uri "http://smartdevice.local/data"

# Specific field

(Invoke-RestMethod -Uri "http://smartdevice.local/data").temp

## 4.6 Arduino / ESP32 (another device acting as a client)

#include <HTTPClient.h>

HTTPClient http;

http.begin("http://smartdevice.local/data");

int code = http.GET();

if (code == 200) {

String body = http.getString();

Serial.println(body);

}

http.end();

# 5. Limits & Best Practices

| Limit | Value / Recommendation |
| --- | --- |
| Recommended polling interval | 1 – 2 seconds |
| Maximum useful polling rate | 1 Hz (sensors do not update faster) |
| Maximum simultaneous clients | 4 |
| Request timeout (suggested) | 3 seconds |
| Persistent connections | Not required; each GET can be a fresh request |

To minimize load on the ESP32 when running multiple clients, prefer a poll interval of 2 seconds or more and reuse a single HTTP session per client rather than opening a new TCP connection for every request.

# 6. Versioning

This document describes API version 1.0. The API does not expose a version header — clients should treat the schema described in Section 2.3 as stable until a new version of this document is issued.